Initial Design Calculation

Ideal boost-converter duty-cycle range

1. Assumptions and governing equation

Assumptions. Ideal components, continuous-conduction mode (CCM), and steady-state operation.

For an ideal boost converter operating in continuous-conduction mode (CCM) and steady state, the duty cycle D can be derived using the inductor volt-second balance principle (literature).

The duty cycle is obtained using inductor volt-second balance: the average inductor voltage over one switching period is zero.

| vL = L · diL/dt | (1) |
| --- | --- |

| VL,avg = 0 | (2) |
| --- | --- |

2. Inductor voltage during each switching interval

| Interval | Duration | Inductor voltage | Volt-second contribution |
| --- | --- | --- | --- |
| MOSFET ON | DTₛ | Vᵢₙ | Vᵢₙ · DTₛ |
| MOSFET OFF | (1 − D)Tₛ | Vᵢₙ − Vₒᵤₜ | (Vᵢₙ − Vₒᵤₜ)(1 − D)Tₛ |

3. Apply volt-second balance

| VinDTs + (Vin − Vout)(1 − D)Ts = 0 | (3) |
| --- | --- |

| Vout = Vin/(1 − D) | (4) |
| --- | --- |

| D = 1 − Vin/Vout | (5) |
| --- | --- |

4. Numerical duty-cycle range

For Vin = 2.5–3.5 V, Vout = 5 V:

| Input condition | Calculation | Duty cycle |
| --- | --- | --- |
| Vᵢₙ = 2.5 V | 1 − 2.5/5 | 50% |
| Vᵢₙ = 3.5 V | 1 − 3.5/5 | 30% |

Required ideal duty-cycle range:  30% ≤ D ≤ 50%

5. Output-current range

From the specified output-power range, Pₒ = 0.5–5 W, at Vₒ = 5 V:

| Iₒ = Pₒ/Vₒ ⟹ Iₒ,min = 0.5/5 = 0.10 A; Iₒ,max = 5/5 = 1.00 A | (6) |
| --- | --- |

6. Inductor selection for CCM

The preliminary value is selected so the ideal converter remains in continuous-conduction mode at the minimum specified load. During the MOSFET ON interval:

| ΔIₗ = VᵢₙDTₛ/L = VᵢₙD/(Lfₛ) | (7) |
| --- | --- |

At the CCM/DCM boundary, the minimum inductor current is zero, so ΔIₗ = 2Iₗ,avg. For an ideal boost converter, Iₗ,avg = Pₒ/Vᵢₙ. Therefore:

| Lcrit = VᵢₙD/(2fₛIₗ,avg) = Vᵢₙ²(1 − Vᵢₙ/Vₒ)/(2fₛPₒ) | (8) |
| --- | --- |

Worst-case check over the full input range. Use Pₒ,min = 0.5 W, Vₒ = 5 V and fₛ = 100 kHz. Maximising Lcrit(Vᵢₙ) gives Vᵢₙ = 2Vₒ/3 = 3.33 V, which lies inside 2.5–3.5 V:

| Lcrit,max = (3.33 V)²(1 − 3.33/5)/(2 × 100 kHz × 0.5 W) = 37.0 μH | (9) |
| --- | --- |

| Design input | Value used |
| --- | --- |
| Minimum theoretical value for CCM | L > 37.0 μH |
| Selected standard value | L = 47 μH |
| Inductor current at 0.5 W, 3.33 V | Iₗ,avg = 0.150 A; ΔIₗ = 0.236 A |
| Minimum inductor current | Iₗ,min = 0.150 − 0.236/2 = 0.032 A > 0 |

7. Preliminary current-rating check

At maximum power and minimum input voltage, the ideal peak current is:

| Iₗ,peak = Pₒ,max/Vᵢₙ,min + ΔIₗ/2 = 5/2.5 + 0.266/2 = 2.13 A | (10) |
| --- | --- |

Component recommendation. Use a 47 μH inductor with low DCR, RMS-current rating ≥ 2.5 A, and saturation-current rating ≥ 3 A. These margins allow for converter losses and component tolerances; the final ratings must be verified using the simulated and measured peak current.

8. Output-capacitor selection for voltage ripple

During the MOSFET ON interval, the diode is reverse-biased and the output capacitor supplies the load. Hence:

| ΔQ = IₒDTₛ; ΔVₒ,C = ΔQ/Cₒ = IₒD/(fₛCₒ) | (11) |
| --- | --- |

Worst case. Use Iₒ,max = 1 A, Dmax = 0.50, fₛ = 100 kHz and ΔVₒ,max = 50 mV:

| Cₒ,min ≥ Iₒ,maxDmax/(fₛΔVₒ,max) = 1 × 0.50/(100 kHz × 0.05) = 100 μF | (12) |
| --- | --- |

Because 100 μF uses the entire ideal ripple allowance, select the next larger preliminary value Cₒ = 220 μF to leave margin for ESR, tolerance and non-ideal switching effects:

| ΔVₒ,C = 1 × 0.50/(100 kHz × 220 μF) = 22.7 mV | (13) |
| --- | --- |

Final verification. Including ESR, ΔVₒ,pp ≈ IₒD/(fₛCₒ) + ΔI꜀·ESR. Verify in PLECS that ΔVₒ,pp ≤ 50 mV.